PRINCÍPIOS DE SISTEMAS DE COMUNICAÇÃO

Fourier Transform Example

The impulse response of a linear, time-invariant system is described by the following equation and shown in the accompanying figure.

Impulse Response h(t)

Since h(t) is non-periodic, it can be represented by its Fourier Transform. The transform can be easily computed by using the Table of Fourier Transforms and applying the linearity property. The magnitude and phase of the Fourier Transform for h(t) are shown in the figures. The magnitude is plotted in decibels (db), which is 20*log10(Absolute Value). Decibels are a convenient way of plotting magnitudes which cover a wide range of values. Note that the frequency scale in both plots is a logarithmic scale. Major changes in magnitude and phase generally occur on a log scale rather than a linear scale in most Fourier or Laplace Transforms.

Magnitude Plot for H(w)                                                            Phase Plot for H(w)

Now, we will apply 3 different pure sinusoids as inputs to the system, one at a time. The first will have a frequency of 0.1 r/s. At that frequency, the magnitude of H(w) is 23.3136 (27.3522 db) and the phase is -40.4784 deg. Note the relative magnitudes and phases of the input and output signal at that frequency, and compare them with the magnitude and phase plots in the frequency domain.                                                              Signals at 0.1rd/s

The next input signal has a frequency of 3.16 r/s. The magnitude and phase of the system at this frequency are 2.3771 (7.5209 db) and -69.8790 deg, respectively. Again note the relations between the input and output signals in the time domain and compare them with the information available from the frequency domain plots of H(w).

Signals at 3.16 r/s

The last of the pure, single-frequency input signals has a frequency of 100 r/s. The magnitude and phase of H(w) at this frequency are 0.0800 (-21.9338 db) and -89.6893 deg, respectively. Once again, you should compare the frequency domain curves for H(w) with the actual time domain relations between the input and output signals.

Signals at 100 r/s

Now we will look at an input signal which has a multitude of frequencies in it, namely the unit step signal. The magnitude plot is a straight line when plotted in db vs. frequency on a log scale. The magnitude decreases by a factor of 10 every time the frequency increases by a factor of 10. In terms of db, the slope of the line is -20 db/decade of frequency). The phase for the unit step is constant at -90 degrees for all positive frequencies.

Frequency Domain Magnitude for Step Function

The output signal will now be composed of all frequencies also. The system will respond to each of the frequencies in the input signal just as it did in the previous examples -- multiplying the amplitude and shifting the phase by the magnitude and phase of H(w) at the corresponding frequency. Now there are an infinite number of them, but it works the same way. The response to the individual frequencies are added to produce the output signal.

Step Response of the System

The magnitude of the output signal in the frequency domain is the product of the magnitude of the input signal X(w) and the magnitude of the system H(w). In decibels, the plot of the output magnitude can be obtained by adding the input and system magnitudes, since the log is being taken. The phase of the output Y(w) is the sum of the phases of the input X(w) and the system H(w).

Fourier Transform Magnitude of Step Reponse                                         Fourier Transform Phase of Step Reponse

MATLAB Code

%      ***** MATLAB Code Starts Here *****

%

w = logspace(-3,3,601);		% frequency vector on log scale

th = [0:0.025:50];		% time vector for h(t)

%

h = 5*exp(-2*th) + 3*exp(-0.1*th) + 6*th.*exp(-5*th);	% impulse response

%

numh = 5*conv([1 0.1],[1 10 25]) + 3*conv([1 2],[1 10 25]) + ...

6*[0 conv([1 2],[1 0.1])];

denh = conv([1 2],conv([1 0.1],[1 10 25]));	% Transfer function for h(t)

%

%  The following 1 line of code could be used to generate the magnitude

%  and phase for H(w) use the "bode" function of MATLAB.  However, for this

%  example, the Fourier Transform is shown explicitly.  The "bode" function

%  can be used here since for a function which has a Fourier Transform, the

%  Fourier Transform is equivalent to the Laplace Transform being evaluated

%  along the jw axis.  That is what the "bode" function does.  numh and denh

%  are the numerator and denominator of the Laplace transform H(s).

%

%		[magft,phft] = bode(numh,denh,w);

%

numft = (163.7-92.5*w.^2) + j*w.*(277.6-8*w.^2);	% Fourier Transform

denft = (5-46.2*w.^2+w.^4) + j*w.*(54.5-12.1*w.^2);	% evaluation

%

magft = abs(numft) ./ abs(denft);	% calculate magnitude and phase

phft = (unwrap(angle(numft)) - unwrap(angle(denft))) * 180/pi;

%

figure(1),plot(th,h),grid,xlabel('Time (s)'),ylabel('Amplitude'),...

title('Impulse Response h(t)')

%

figure(2),semilogx(w,20*log10(magft)),grid,xlabel('Frequency (r/s)'),...

ylabel('Magnitude (db)'),title('Fourier Transform Magnitude for H(w)')

%

figure(3),semilogx(w,phft),grid,xlabel('Frequency (r/s)'),...

ylabel('Phase (deg)'),title('Fourier Transform Phase for H(w)')

%

[i1,i2] = min(abs(w - 0.1));		% locate specific frequencies

[i1,i3] = min(abs(w - sqrt(10)));

[i1,i4] = min(abs(w - 100));

%

tx1 = [0:0.1:200];			% input & output at w = 0.1 r/s

x1 = cos(w(i2)*tx1);

y1 = magft(i2) * cos(w(i2)*tx1 + phft(i2)*pi/180);

%

tx2 = tx1/50;				% input & output at w = 3.16 r/s

x2 = cos(w(i3)*tx2);

y2 = magft(i3) * cos(w(i3)*tx2 + phft(i3)*pi/180);

%

tx3 = tx2/20;				% input & output at w = 100 r/s

x3 = cos(w(i4)*tx3);

y3 = magft(i4) * cos(w(i4)*tx3 + phft(i4)*pi/180);

%

figure(4),plot(tx1,x1,tx1,y1),grid,xlabel('Time (s)'),ylabel('Amplitude'),...

title('Input x(t) and Output y(t) at 0.1 r/s'),gtext('x(t)'),gtext('y(t)')

%

figure(5),plot(tx2,x2,tx2,y2),grid,xlabel('Time (s)'),ylabel('Amplitude'),...

title('Input x(t) and Output y(t) at 3.16 r/s'),gtext('x(t)'),gtext('y(t)')

%

figure(6),plot(tx3,x3,tx3,y3),grid,xlabel('Time (s)'),ylabel('Amplitude'),...

title('Input x(t) and Output y(t) at 100 r/s'),gtext('x(t)'),gtext('y(t)')

%

[magx,phx] = bode(1,[1 0],w);		% Magnitude & Phase for Step Input

%

ys = step(numh,denh,th);			% Output for Step Input

yss = numh(4) / denh(5);			% steady-state value for output

%

magys = magft .* magx';			% Output magnitude and phase

phys = phft + phx';

%

figure(7),semilogx(w,20*log10(magx)),grid,xlabel('Frequency (r/s)'),...

ylabel('Magnitude (db)'),title('Magnitude for Step Function')

%

figure(8),plot(th,ys,[0 50],[yss yss],'w--'),grid,xlabel('Time (s)'),

ylabel('Amplitude'),title('Step Response of System'),...

gtext('Dashed line is the steady-state value of output')

%

figure(9),semilogx(w,20*log10(magys),'y',w,20*log10(magx),'g',...

w,20*log10(magft),'r'),grid,xlabel('Frequency (r/s)'),

ylabel('Magnitude (db)'),

title('Fourier Transform Magnitude for Step Response'),gtext('X(w)'),...

gtext('H(w)'),gtext('Y(w)')

%

figure(10),semilogx(w,phys,'y',w,phx,'g',w,phft,'r'),grid,...

xlabel('Frequency (r/s)'),ylabel('Phase (deg)'),...

title('Fourier Transform Phase for Step Response'),gtext('X(w)'),...

gtext('H(w)'),gtext('Y(w)')

%

%      ***** MATLAB Code Stops Here *****

TAREFA: Refaça o exemplo para h(t) = (e-t - 0,1.e-10t).u(t). Utilize sinusoides de freqüências 0.1rd/s, 1rd/s, 5rd/s, 10rd/s e 100rd/s. Repita o mesmo procedimento para a resposta ao degrau unitário

Data da Entrega: 24/04/2000